Annex F

Central Area Office

Lot#1.1. A to 1.4. A

ITB-AFG-AFC-010-2026 – Supply and Delivery of Water Supply System (Equipment and Materials)

List of required items for sample check bidders must fill out and complete this Annex F to provide proof of the availability of the following items, including locations and current stock levels.

Lot#1.1.A- Construction Materials

| S/N | Items | Specification | Location and Current Stock Levels | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Isogam | Supply and installation Isogam polymer, modified bitumen waterproofing membrane, 4 mm thick, reinforced with polyester and fiberglass, UV-resistant. The quality should be equal to Sanat Bam, Weight: Approximately 37–42 kg per roll |  |  |
| 2 | Steel | Steel Reinforcement Bars Grades 60 which has yeild strength of 60,000 psi with deferent sizes 18mm The steel must pass the bend test requirements as prescribed in the referenced ASTM A615 standard. Test specimens shall withstand being bent around a pin (mandrel) without cracking on the outside radius. The diameter of the pin and the angle of the bend (typically 90° or 180°). |  |  |
| 3 | Brick | Brick (first grade) including transportation and the dimention should be 7cm ,10cm ,20 cm |  |  |
| 4 | Oil Paint | Oil paint with its application that is equal to the paint of Khorasan it should be at least 3 layers |  |  |
| 5 | Plastic Paint | Plastic Paint with workmanship (100%) according to afghan faizi color |  |  |

Lot# 1.2.A- Pipe and fittings

| S/N | Items | Specification | Location and Current Stock Levels | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Pipe PVC 4 inch | Pipe PVC 4 inch that quality equal to Clear-Guard schedule 40 High Pressure Pipe class C 9bar |  |  |
| 2 | 1.5" PP-R (Polypropylene Random Copolymer) pipes | 1.5" PP-R (Polypropylene Random Copolymer) pipes, should be designated as DN40 or 50mm, thermoplastic pipes used for hot/cold water, heating, and HVAC and typically feature an outer diameter of 48.3mm–50mm and 3.5mm wall thickness |  |  |
| 3 | 3-inch Galvanized Iron (GI) elbow | 3-inch Galvanized Iron (GI) elbow with a heavy-duty, corrosion-resistant pipe fitting, typically featuring a 90-degree angle and standard BSP threading for secure, high-pressure, and durable connections in both residential and commercial plumbing systems |  |  |
| 4 | 2-inch Galvanized Iron (GI) pipe | 2-inch Galvanized Iron (GI) pipe with, durable, zinc-coated steel pipe (50mm nominal bore) commonly used for water supply, structural frameworks, and industrial applications |  |  |
| 5 | 90 mm (3 inch) PE 100 PN10 HDPE pipe | 90 mm (3 inch) PE 100 PN10 HDPE pipe with high-density polyethylene pipe, frequently used for water supply, offering 10 bar pressure resistance. Commonly manufactured to SDR 17 or 13.6, it has a wall thickness of approximately5.4 mm to 6.7 mm and is ideal for potable water, featuring corrosion resistance, flexibility, and longevity |  |  |
| 6 | 32 mm (1inch) PE 100 PN10 HDPE pipe | 32 mm (1inch) PE 100 PN10 HDPE pipe with high-density polyethylene pipe, frequently used for water supply, offering 10 bar pressure resistance. Commonly manufactured to SDR 11 , it has a wall thickness of approximately 3. mm and is ideal for potable water, featuring corrosion resistance, flexibility, and longevity |  |  |
| 7 | 3-inch cast iron flanged gate valve, | 3-inch cast iron flanged gate valve, is a heavy-duty, usually Class 125/150 rated component suitable for water, oil, and gas applications. These valves feature cast iron bodies with either rising or non-rising stems, and they connect to piping systems using flange adapters to ensure a leak-resistant, flexible seal. Key Components & Specifications: Valve Size: 3 Inch. Body Material: Cast Iron (commonly ASTM A126 Class B or ASTM A536 ductile iron) for high strength. Connection: Flanged, often compliant with ANSI B16.1 Class 125/150 or PN10/PN16 standards. Flange Adapters: Typically cast or ductile iron, featuring an epoxy-coated body for durability and a compression ring to allow connection between flanged valve ends and plain-ended pipes. Operating Mechanisms: Available with handwheels (manual operation) or Operating Nuts (Op Nut) for underground utility access. Working Pressure: Generally to CWP (Cold Working Pressure |  |  |
| 8 | Steel Tap 0.5" | Steel Tap 0.5" |  |  |
| 9 | PE male Adaptor 90 mm x 3", PN-16 | PE male Adaptor 90 mm x 3", PN-16 |  |  |

Lot#1.3. A- Solar system

| S/N | Items | Specification | Location and Current Stock Levels | Remarks |
| --- | --- | --- | --- | --- |
| 1 | A 12x12 waterproof junction box | A 12x12 waterproof junction box is commonly a PVC or ABS plastic enclosure, typically IP65 or IP67 rated, offering protection against rain, snow, dust, and UV light for outdoor and indoor electrical connections |  |  |
| 2 | Ac breaker | Ac breaker: DZ47 1P D Curve Motor AC Circuit Breaker 10A~125A 400V Start Switch Machine Industrial Equipment Water Pump high Current(125A |  |  |
| 3 | PV Disconnect Switch | PV Disconnect Switch :Types: Can be a standalone rotary disconnect, a breaker within a combiner box, or built into the inverter. Specifications: Often rated for \(1000\V DC) or higher and 32Aor more, with enclosures rated for outdoor, waterproof use (IP65). Components: four-pole switches, which can be configured to act as double-pole switches for increased safety and to handle higher voltages by breaking the connection across multiple contacts. |  |  |
| 4 | Standard Pipe Clamp | Standard Pipe Clamp CLIC Top 59-63mm M8 for Steel/Metal/Drain Tube |  |  |

Lot#1.4.A- Drilling and testing

| S/N | Items | Specification | Location and Current Stock Levels | Remarks |
| --- | --- | --- | --- | --- |
| 1 | PVC casing pipe | PVC casing pipe with diameter of 8'' Class D. The qaulity should be equal to High Standard pipe Supply and installation of 8-inch diameter Class D PVC casingpipe, including PVC solvent cement (adhesive), threaded connections (couplings/screws as required), and all necessary accessories for proper installation. The materials shall be of high standard quality, equivalent to approved international specifications, and suitable for water well applications.The qaulity should be similar to High Standard pipe |  |  |
| 2 | Gravel | Gravel back filling around filter (gravel packing) from river gravel cleaned in crush plant, and the size of gravel will be considered according to the geological formations grain size. |  |  |

Note: This Annex F must be signed by the Suppliers, confirming that the listed items are available at their respective location and indicating the offered quantity of each item they can provide.

Sign: _________________

Date: _________________

Stamp: ________________